UNDERGRADUATE THESIS TECHNICAL REPORT

School of Engineering and Applied Science

University of Virginia

Branch and Confidence Prediction

Using Perceptrons

Submitted by

Stephen Kelley

Computer Science

TCC 402

Section 6 (2 p.m.)

April 3, 2001

On my honor as a student, I have neither given nor received unauthorized aid on this paper as defined by the Honor Guidelines for Papers in TCC Courses.

Signed______________________________

Approved_________________________________		Date____________________

Kevin Skadron – Technical Advisor

Approved_________________________________		Date____________________

W. Bernard Carlson – TCC Advisor

# Abstract

The improvement of branch prediction remains an important topic in computer science, as it correlates directly to computer performance.  Branch predictor accuracy is critical because mispredictions stall the fetch engine and starve the execution engine.  A "mere" 5% misprediction rate can cut performance by as much as 20-30% in today's wide-issue processors.  Current branch prediction methods are experiencing diminishing returns, and a new approach may prove more successful.  Thus, I am investigating using the perceptron, a simple artificial neural network, to perform branch prediction.  Perceptrons are excellent pattern recognizers, and seemed well suited to performing branch prediction.  Due to their design, a prediction confidence factor comes out of the calculation at no additional computation cost.  However, extreme branch misprediction rates and poor confidence prediction make the perceptron vastly inferior to the conventional methods I have studied in this thesis project.

Table of Contents

Abstract	ii

Table of Figures	iv

1	Why perceptrons?	1

1.1	Branches	1

1.2	Branch Prediction	2

1.3	Neural Networks	3

1.4	Multipath Execution	3

2	Introduction to Branches and Perceptrons	5

2.1	Branches	5

2.2	Branch Prediction	6

2.3	Neural Networks	12

2.4	Perceptron-based Branch Prediction	15

2.5	Confidence Prediction	16

3	Design	19

3.1	Weight Table	19

3.2	Global Branch History Register	20

3.3	Branch Prediction	20

3.4	Learning	21

3.5	Example	22

4	Results	24

4.1	Simulation Methodology	24

4.2	Data	27

4.3	Analysis	30

5	Conclusion	33

5.1	Summary	33

5.2	Interpretation	33

5.3	Recommendations	36

Bibliography	37

Appendix A – Implementation	A-1

A.1 Initialization of Data Structures	A-1

A.2 Prediction Calculation	A-1

A.3 Direction Calculation	A-2

A.4 Updating Functions	A-2

A.4 Confidence Prediction	A-3

Appendix B – Additional Data	B-1

B.1 Misprediction Rates	B-1

B.2 Confidence Prediction	B-3

Table of Figures

Figure 1:  A generic pipeline without branch prediction	6

Figure 2:  Pipeline behavior with branch prediction	7

Figure 3:  The state diagram of a saturating counter.	8

Figure 4:  An example of local history prediction.	10

Figure 5:  An example of global history prediction.	11

Figure 6:  An example of a hybrid predictor.	12

Figure 7:  An example of a linearly separable branch	15

Figure 8:  Graphical model of a perceptron.	16

Figure 9:  A schematic of perceptron-based branch prediction.	19

# Why perceptrons?

Improving branch prediction, although an already well-studied subject in computer science, remains important because delivering very high branch-prediction rates is crucial to further performance gains in high-performance computing.  In today’s modern processors, even small misprediction rates can severely harm computer performance [19] .

Current branch prediction approaches have entered a domain of diminishing returns.  There is a need to find alternative methods of branch prediction to counter these impasses.  Because their implementation is different than traditional branch prediction techniques, neural networks are a promising alternative to these problems.

A neural network is a group of independent processing elements, collectively working and interacting together.  They are excellent pattern recognizers, and seem well suited to undertaking branch prediction.  Using neural networks to perform branch prediction has been overlooked in the computer science field, and little research is available on this subject.

## Branches

Branches are instructions that can change the flow of a program’s execution.  These correspond to high-level language constructs like if statements and for and while loops.  Branches create a performance bottleneck because, without prediction, the processor must stall until the branch completes execution and the correct path is known (e.g. the then or else clause of the if statement).

## Branch Prediction

Branch prediction involves predicting which branch path the program will continue.  This is accomplished by tracking the patterns the branch has taken before.  The branch predictor analyzes these patterns and determines whether a given branch will be taken or not taken based on its previous behavior.  Future instructions and data can now be pre-fetched from memory and made immediately available for execution.  This reduces the time the CPU spends stalled, thus improving processor performance.

To gain information on prior branch outcomes, the branch predictor inspects a branch history register.  These can either be local, i.e. a particular branch’s history, or global, in which a portion of the entire program’s flow is recorded.  Both have advantages and disadvantages.  Because local branch history registers only store a particular branch’s information, they do not get any other branch’s data mixed in with their own, and are effective at analyzing repeating patterns such as TNTNTN.  A side effect of local history prediction is that it is unable to uncover correlation among branches.

There are also hybrid models of branch prediction, which make use of a meta predictor.  A meta predictor is used to select the method of branch prediction the processor will use.  This benefits the branch predictor because it uses the method which should be best suited to predict a particular branch.  The main obstacle with this approach is setting the meta predictor correctly.  This raises the complexity of the branch predictor as a whole because now not only does it have to predict the correct direction the branch will take, but in addition, has to also determine the predictor that will yield the best results.

## Neural Networks

Neural networks are sets of many independent interconnected processing elements, called perceptrons, that collectively work and interact as one unit.  Such a network can be trained by supplying data in which the outcome is already known, usually in the form of a Boolean equation.  As the neural network processes the data, weights are assigned to edges between the nodes.  These weights correspond to the influence that each input has on the final output, and are adjusted as the program runs.  Ultimately, the network learns the solution itself [3] .

Neural networks are excellent pattern recognizers, because the output is a direct correlation with the inputs, given by a simple mathematical formula [6].  They are capable of sorting through large amounts of data, accepting only relevant inputs by adjusting the weights of inputs.

## Multipath Execution

Many programs suffer a substantial number of mispredictions.  Since the delays caused by conditional-branch mispredictions remain a serious problem, multipath execution has been examined to reduce the need for branch prediction.  Multipath execution is the simultaneous execution of both the taken and not taken branches following the conditional branch, with cancellation of the erroneous prediction when the branch has finally been resolved. [1]

Multipath execution requires the existence of a confidence predictor, which attempts to evaluate the likelihood that a given branch prediction will be correct.  Confidence prediction is necessary in multipath execution because otherwise the program will blindly fork down all possible branches.  Because of finite memory sizes, the processor will run out of room for new paths, and be unable to fork when an important branch comes along.  Instead, the processor can combine its resources by trying to predict when to fork and when not to fork.

Current confidence predictors require additional processor and memory resources to both calculate and store the confidence predictor.  However, perceptron-based branch prediction gives a confidence predictor that will come at no additional cost to the system once the branch direction has been predicted.

# Introduction to Branches and Perceptrons

## Branches

In almost all instruction sets, branch instructions have exactly two possible outcomes: not taken, the sequential case in which the program continues executing the instructions that immediately follow the branch, and taken, the non sequential case in which execution jumps to a target specified in the branch instruction [21].  For example, in the case of an if statement, these two outcomes correspond to the then clause or the else clause.  For for or while loops, the two cases are the iteration of the loop body or the termination of the loop altogether.

Branches create a problem because the identity of the proper path can only be known after the branch has finished testing its condition, a process that adds latency to the pipeline [21].  This latency occurs because the branch direction is not computed until after the execute stage.  However, new instructions cannot be fetched until the branch is resolved because the processor does not know which instructions it will execute next.   Figure 1 shows an example picture of the instruction pipeline without branch prediction.  Note the 2-cycle bubble that appears.

Figure 1:  A branch flowing through a generic pipeline without branch prediction.  The branch, the first gray box is fetched in the first cycle, but no other instructions can be issued until the branch is executed [21].

## Branch Prediction

In order to reduce this branch resolution latency, processors have added branch predictors, which attempt to predict the direction of branch before it is actually resolved.  New instructions down the predicted path can now be fetched and speculatively executed.  Figure 2 shows another example pipeline, this time with branch prediction.  When compared with the pipeline in Figure 1, we see that the pipeline bubble has been effectively eliminated from the pipeline due to branch prediction.  Upon a correct prediction, these instructions will become non-speculative and execute after the branch.  If the branch is mispredicted, these instructions will be squashed and instructions down the correct branch path will be fetched [21].  It is worth noting that the processor is no worse off in the case of a misprediction, as it would have spent the branch resolution time idle anyway.

Figure 2:  Pipeline behavior with branch prediction.  While the branch, (shown in gray) is being resolved and executed, new instructions can be fetched and begun to be executed at the same time [21].

### Bimodal Prediction

The simplest form of branch prediction is bimodal prediction, proposed by Smith [23], which makes use of a table of saturating bit-wise counters, called a pattern history table, PHT.  The PHT has one entry per branch, and is indexed by the branch address.  Entries in the PHT are saturating counters of arbitrary length, and are incremented following a taken branch, and decremented following a not taken branch.  For a two-bit counter, values of 00 and 01 (i.e. 0 and 1 in decimal notation) signify a not taken prediction, while values of 10 and 11 (2 and 3 in decimal) signify a taken prediction.  Once a branch is encountered the appropriate entry in the PHT is accessed using the branch address, and the prediction is made [21].  Once path of the branch is known, the counters are updated.  Figure 3 shows the state diagram of a simple bimodal predictor.

Figure 3:  The state diagram of a saturating counter.

The most common and effective counter length is two bits.  Varying the amount of bits used by the counter leads to significant changes in prediction behavior.  Shorter history lengths are much quicker to adapt to changes in program behavior, but can suffer from a substantial number of mispredictions due to their lack of history.  For instance, one-bit counters will mispredict every time the branch changes direction.  Longer lengths give more history and do not suffer from as many mispredictions, but too many bits make it difficult for the predictor to learn legitimate changes in program behavior [21].  In the case of a context switch, in which the processor switches from one procedure to another, a branch may change its behavior, and longer history lengths will take much more time to relearn this new valid branch behavior.

### Local History Prediction

Bimodal prediction cannot identify branch patterns.  One such solution to this problem are two level predictors, proposed simultaneously by Yeh & Patt [25] and Pan, So, & Rahmeh [16].  These methods track branch patterns are tracked and different predictions are made for the same branch based on its recent history.  One such scheme is local history prediction, proposed by Yeh & Patt [25].  Here a table of shift registers, called a branch history table (BHT), track of the outcomes of the each branch.  To make its prediction, the predictor looks up the current branch’s entry in the BHT using the branch address.  This entry is used as the index into the PHT of two-bit counters, and the prediction is made.  A benefit of this scheme is that it allows common branch patterns, like alternating branches (TNTNTN…) to be correctly predicted, as well as irregular but repeating patterns such as TTNTTN [21].  An example of a local history predictor appears in Figure 4.

Figure 4:  An example of local history prediction.

### Global History Prediction

Alternatively, the BHT can be replaced with a single, global shift register, the global branch history register or GBHR [21].  Outcomes of all branches are recorded into GBHR.  The GBHR is then used as the index into the PHT.  A benefit of global history prediction is that it is able uncover correlation among branches.  However, two branches may share the same recent history but have unrelated behavior.  In this system, both branches will share the same slot in the PHT, called aliasing, and leads to the branches overwriting one another’s states.  Combining the history bits with some bits from the branch's address can alleviate this problem [21].  One simple way, proposed by McFarling [15], to do this is to XOR the two bit strings together, thus creating a unique entry for both the branch and the recent global history.  Figure 5 shows an example of global history prediction.

Figure 5:  An example of global history prediction.

### Hybrid Prediction

Some branches can benefit from local history prediction, while others benefit more from global history prediction.  Using only one predictor can thus always be harmful to some types of branches.  A solution to this problem is to use a hybrid predictor, proposed by McFarling [15] which instead of predicting branch outcomes predicts which predictor will be best suited to the branch.  This can be done statically [8], with branches assigned at compile time, or dynamic, in which the predictor tracks the success of the predictors.  This benefits the branch predictor because it is using the method that should best suited to predict a particular branch.  However, the main obstacle to overcome with this approach is setting the meta predictor correctly.  This raises the complexity of the branch predictor as a whole because now not only do you have to predict the correct direction the branch will take, but in addition, have to also predict the predictor that will yield the best results.  An example of a hybrid predictor appears in Figure 6.

Figure 6:  An example of a hybrid predictor.

### Aliasing

Ideally, all branches have a unique entry in the BHT and PHT.  However, these tables have a finite size.  A hash function must therefore be used to index into the table.  A hash function divides the branch address is by the size of the table and the remainder becomes the index into the PHT.  This is called modulo hashing.  Unfortunately, some branches end up hashing to the same entry.  Sometimes this aliasing is harmless, or even beneficial, as branches can communicate additional information.  However, mostly it leads to destructive interference as branches overwrite one another [21].

## Neural Networks

Artificial neural networks were originally developed as tools for the exploration and reproduction of the way a mammalian brain operates and learns.  They offer ideal solutions to such human information processing tasks as speech, vision, olfaction, touch, knowledge processing and motor control. Today, most research is directed towards the development of artificial neural networks for applications such as data compression, optimization, pattern matching, system modeling, function approximation, and control.  In practice, neural networks are especially useful for mapping problems which are tolerant of a high error rate, have lots of example data available, but to which hard and fast rules cannot easily be applied.

### Learning Algorithms

Neural network learn using either supervised or unsupervised learning algorithms.  In supervised learning, the correct results (target values, desired outputs) are known and are given to the neural network during training so that the neural network can adjust its weights to try match its outputs to the target values. After training, the neural network is tested by giving it only input values, not target values, and seeing how close it comes to outputting the correct target values.  In unsupervised learning, the neural network is not provided with the correct results during training. Unsupervised neural networks usually perform some kind of data compression, such as dimensionality reduction or clustering [17].

### Data

The two major kinds of data neural networks accept are categorical and quantitative.  Categorical variables take only a finite number of possible values, and there are usually several or more cases falling into each category. Categorical variables may have symbolic values (e.g., "male" and "female", or "red", "green" and "blue").  Quantitative variables are numerical measurements of some attribute, such as length in meters. The measurements must be made in such a way that at least some arithmetic relations among the measurements reflect analogous relations among the attributes of the objects that are measured [17].

### Linear Separability

Given enough training time, neural networks can predict any linearly separable branch with 100% accuracy.  A linearly separable branch is one in which a single line forms a boundary between the two potential outcomes (e.g. “taken” and “not taken”).  Figure 7 shows an example of a linear separable branch.  However, a neural network can never learn a linearly inseparable branch with 100% accuracy, no matter how long it is trained.  By contrast, twolevel PHT schemes can learn any Boolean function if given enough training time [11].  It is worth noting that even though the perceptron cannot predict linearly inseparable branches with 100% accuracy, they still can be an excellent predictor for these branches.

Figure 7:  An example of a linearly separable branch, with (+) being taken outcomes and (-) being not taken outcomes.

## Perceptron-based Branch Prediction

Dynamic branch prediction with perceptrons was first proposed by Jimenez and Lin [11].  Figure 8 shows a graphical model of a sample branch predicting perceptron.  The input values x1 through xn are prior branch outcomes coming from the global branch history register.  These are bipolar; each xi is either 1, meaning the branch was taken, or

–1, in the case which the branch was not taken.  Weights w1 through wn are weights associated with their respective input, the larger the absolute value of wi, the higher degree of correlation of xi has with the output.  These values come from a table of weights, indexed by the branch address.  The output, y, is computed as the dot product of these weighted input vectors.

Figure 8:  Graphical model of a perceptron.

## Confidence Prediction

Many programs suffer a substantial number of mispredictions.  Since the delays caused by conditional-branch mispredictions remain a serious problem, multipath execution has been studied.  Multipath execution is the simultaneous execution of both the taken and not taken branches following the conditional branch, with cancellation of the erroneous path when the branch has finally been resolved [1].

Multipath execution requires the existence of a confidence predictor, which attempts to evaluate the likelihood that a given branch prediction will be correct.  Confidence prediction is necessary in multipath execution because without it the program will blindly fork on all possible branches.  This will lead to the program running out of room for new paths, and then be unable to fork when an important branch comes along.  Instead, we can conserve our resources by trying to predict when to fork and when not to fork.

Current confidence predictors require additional processor and memory resources to both calculate and store the confidence predictor.  However, percerptron-based branch prediction gives a confidence predictor that will come at no additional computational or storage cost to the system once the branch direction has been predicted.

### Statistical Analysis of Predictors

In studying confidence prediction, branches are classified either as high confidence or low confidence according to a set threshold.  If the branch’s attributes are above this threshold, it is considered high confidence, and low confidence otherwise.  These classifications are further divided based on whether or not the branch was predicted correctly, producing four variables – correct high confidence (CHC), incorrect high confidence (IHC), correct low confidence (CLC), and incorrect low confidence (ILC).

Examination of these variables produces statistics on the overall effectiveness of the confidence predictors.  This thesis project will study four measures of effectiveness [7]: sensitivity, predicted value of a positive test (PVP), specificity, and predicted value of a negative test (PVN).  The sensitivity is defined as the fraction of correct predictions correctly identified as high confidence, while the specificity is the reverse – the fraction of incorrect predictions correctly identified as low confidence.  These variables define the coverage of the confidence predictor, or how effective the confidence predictor is at classifying branches.  The PVP is the probability that a high confidence prediction is correct, while the PVN is the probability that a low confidence prediction is correct.  The PVP and PVN show how accurate the confidence predictor is.

### Confidence Predictors

As earlier stated, a confidence predictor needs a threshold against which to make a prediction.  This threshold will vary based on the type of predictor used.  This thesis project will examine three types of confidence predictors: agreement, branch clustering, and perceptron-based.  Agreement predictors classify branches based on the agreement of a two-level and bimodal predictor.  If the predictors predict the same direction, the branch is considered high confidence.  If the predictions differ, it is low confidence.  Research  [10] has shown that branch mispredictions often are clustered, with many mispredictions happening in a short amount of instructions, after which the program returns to a steady state.  Jacobsen, Rotenberg, & Smith proposed a method by which the branch is assigned a high or low confidence prediction based on its instruction distance from the last misprediction.  Perceptrons have a unique structure when compared to other branch predictors, in that their prediction comes in the form of a signed integer.  The magnitude of this integer can then be compared to the threshold to obtain a confidence prediction for free.

# Design

Figure 9: A schematic of perceptron-based branch prediction.

## Weight Table

The weight table is the heart of perceptron branch prediction.  This structure holds the weights that allow the perceptron to “learn” branch behavior.  In structure, the weight table is a two-dimensional array, with each row corresponding to a different branch, except of course in cases of aliasing.  At each entry in the row is a weight, an integer whose value dictates how strongly the current branch correlates with a corresponding entry in the global branch history register.    The ith entry in each row of the weight table maps to the ith entry in the GBHR.  The set of weights act like a local mask of the GBHR.  W0 is reserved for the current branch itself, and gives the perceptron a bias input, a measure of local history.

There is also the existence of a threshold for the perceptron.  This is a parameter to the perceptron to decide when enough training has been done, as evidenced by the fact that the output product has become very strong.  Entries in the weight table are updated only upon a misprediction or if the value of the perceptron output is less than the threshold [11].

## Global Branch History Register

The GBHR is a shift register that tracks the last x branches’ outcomes, where x is the history length.  Each time a branch is encountered, its outcome is stored in GBHR1, and the other values are shifted down the array.  The value of GBHRi is the outcome of the branch that occurred i branches ago.  For instance, GBHR5 is the outcome of the branch 5 branches ago.  The outcomes are bipolar, meaning the branch 1 if the branch was taken, -1 if the branch was not taken.  GBHR0 is always 1, allowing the bias input, weight[0] to pass through to the dot product unmanipulated.  The size of the array is the history length + 1, for the bias input of the weight table.

## Branch Prediction

When a branch is encountered, a specific row in the weight table is accessed using the branch’s address.  The prediction is calculated by taking the dot product between this row and the GBHR.  The branch predictor then predicts taken for a positive outcome, and not taken for a negative outcome.  If xi is the value at GBHR[I] and wi is the value at weight[i], then the outcome y can be given by the simple formula:  y =  xi + wi, where i iterates from 0 to them history length –1.  An algorithm to perform the branch prediction is shown below.

sum = 0;

for (i = 0 to history length)

sum += Wi * GBHRi;

if (sum > 0)

return taken;

else

return not taken;

## Learning

A neural network’s main asset is its ability to learn new data.  For the perceptron to learn, after the prediction is made, the data structures must be updated.  Recall updating is only done upon a misprediction or if the output value of the perceptron is less than or equal to the threshold.  Each entry in the weight table is incremented if its corresponding GBHR entry agrees with the prediction and decremented if it does not.  If t is the outcome of the branch (1 for taken, -1 for not taken), xi is the corresponding entry in the GBHR, then the product of the two can simply be added to the existing weight table entry.  Because both the GBHR and the outcome of the branch are bipolar, multiplying these will return an 1 if they agree, and a  -1 if they disagree.  Adding this value to the existing weight table entry will increment or decrement the value appropriately.  An example algorithm to update the weight table is shown below.

if (misprediction || (perceptron_output < perceptron_threshold))

for (I = 0 to history length)

Wi += t * xi

To update the GBHR, the current values are shifted up and the branch direction is stored into GBHR1.  Implementing the shift function can be done by this sequence of code:

for (i = 0 to history length - 1)

GBHRi+1 = GBHRi

GBHR1 = branch_outcome;

## Example

Suppose you are using a history length of 8, and the last 10 branches were TNTTNTNT.  Then the GBHR will look like this:

| 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Also, suppose the weight table entry location indexed by branch address looks like:

| 4 | 15 | -1 | -9 | 29 | 22 | -13 | -5 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Using Jimenez & Lin’s calculation of 1.93 * history length + 14 for the optimal threshold value, the threshold is 29.

### Prediction

To make its prediction, the perceptron now takes the dot product of these two structures.  In mathematical terms, the output y =  wi + xi, where wi is the ith entry in the weight table and xi is the ith entry in the GBHR.  For this example the calculation is as follows:

y = 4*1 + 15*1 + -1*-1 + -9*1 +29*1 + 22*-1 +-13*1 + -5*-1 + 18*1

y = 28

This output is positive, and thus the perceptron predicts the branch is taken.

### Updating

After the branch the processor has resolved the branch direction, it is time to update the data structures.  First we check the conditions.  We did not mispredict, but the output is less than the perceptron threshold.  Thus we update the table.

A weight table entry is incremented if the branch direction agrees the corresponding entry in the GBHR, and decremented otherwise.  Say the perceptron was correct in its prediction and the branch was taken.  The weights are now:

W0 = 4 + 1 * 1 = 5			W1 = 15 + 1 * 1 = 16

W2 = -1 + -1 * 1 = -2			W3 = -9 + -1 * 1 = -8

W4 = 29 + 1 * 1 = 30 			W5 = 22 + -1 * 1 = 21

W6 = -13 + 1 * 1 = -12		W7 = -5 + -1 * 1 = -6

W8 = 18 + 1 * 1 = 19

The resulting structure looks like:

| 5 | 16 | -2 | -8 | 29 | 21 | -12 | -6 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

The new branch outcome is shifted into GBHR1, and the resulting structure looks like:

| 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Results

## Simulation Methodology

### Simulator

All code modifications were made to bpred.c, on the SimpleScalar v.3.0 toolkit [2].  Sim-bpred.c and bpred.h were also modified to reflect the changes made in bpred.c  Executables were compiled for x86 machines.

### Compute Engines

Simulations were run on the LAVA [20] nodes in the computer science department, which are 10 dual-CPU Pentium-III/466 MHz simulations engines using the Linux FreeBSD operating system.

### Benchmarks Used

The following benchmarks were simulated for 1,000,000,000 instructions.  Some benchmarks experience a “warm-up” phase in which behavior is erratic and unrepresentative of the program as a whole.  In this case, simulations are run in a fast mode to reach the chosen simulation window.  Fast forward intervals were chosen based on Skadron’s prior research [21].

- SPECint95

  - 129.compress	fast forward 1648000000

  - 126.gcc		fast forward 0

  - 099.go		fast forward 925000000

  - 134.perl		fast forward 600000000

- Splash2

  - Radiosity	fast forward 300000000

- Other

  - Gnuchess	fast forward 0

  - Wolf 		fast forward 0

### Topics Researched

- Predictor Accuracy

  - Mean misprediction rate of predictors with equal hardware budgets

  - Perceptron Misprediction Rate vs. Table Size

- Confidence Prediction

  - Sensitivity

  - Predicted Value of a Positive Test

  - Specificity

  - Predicted Value of a Negative Test

### Predictor Configurations

#### Hardware Budget Calculation

To calculate the number of bits used by each predictor in determining the hardware budget, I used the following formulas:

- Perceptron – Bits = Entries * History Length * 1 + log2 (Threshold)

- Two-level – Bits = Entries * 2

- Hybrid – Bits = (Two-level Size + Bimodal Entries + Meta Entries) * 2

#### Predictor Accuracy

To compare predictors of equal hardware budgets, I used the following predictor configurations.  The two-level predictor used was a GAg global predictor, in which the PC makes no contribution to the input of the PHT.

- 8K bit Hardware Budget

Predictor			Table Size  	History Length

  - Perceptron		32		22

  - Perceptron		64		12

  - Perceptron		128		8

  - Two-level		4096		5

  - Hybrid

    - Two-level	2048		4

    - Bimodal		1024		N/A

    - Meta		1024		N/A

- 64K bit Hardware Budget

Predictor			Table Size  	History Length

  - Perceptron		256		36

  - Perceptron		512		34

  - Perceptron		1024		9

  - Two-level		32768		8

  - Hybrid

    - Two-level	16384		8

    - Bimodal		8192		N/A

    - Meta		8192		N/A

To compare predictors of equal table sizes, I used the following predictor configurations:

- 8K bit Hardware Budget

Predictor			Table Size  	History Length

  - Perceptron		4096		62

  - Perceptron		4096		5

  - Two-level		4096		5

- 64K bit Hardware Budget

Predictor			Table Size  	History Length

  - Perceptron		32768		62

  - Perceptron		32768		8

  - Two-level		32768		8

Threshold values are set at 1.93 * history length + 14 based on optimal levels given by Jimenex & Lin [11].

#### Confidence Prediction

To compare the different confidence predictors, I used the following predictor configurations:

Predictor			Threshold

- Agreement Predictor	N/A

- Branch Clustering		2 branches

- Branch Clustering		4 branches

- Branch Clustering		7 branches

- Perceptron			1/3 of perceptron threshold

- Perceptron			1/2 of perceptron threshold

- Perceptron			2/3 of perceptron threshold

## Data

### Misprediction Rate

### Confidence Prediction

## Analysis

### Misprediction Rates

As we can see from the data, the perceptron faired far worse than the hybrid and two-level predictors.  This is in direct contradiction to the results in the Jimenez & Lin paper [11], which said the perceptron was better at all hardware budgets above 1 KB.  Unfortunately, I could not reproduce their results.

At both hardware budgets, the perceptron did significantly poorer than the two-level and hybrid predictors.  This is most probably linked to the destructive interference  that occurred as a result of the perceptron’s small table size.  Even at largest table size configurations, the two-level and hybrid predictors had 32x the entries.  Thus they would suffer less from aliasing.

Hardware budgets aside, I also simulated equal entry sizes of the perceptron and the two-level predictors.  Again, the perceptron had a higher misprediction rate.  However, it did perform better than the other perceptron configurations at lower hardware budgets.

Another observation is that the perceptron benefits greatly from increasing table size.  More entries in the table mean that less aliasing occurs, and thus the prediction rate will increase.  You can see from the graph of Misprediction Rate vs. Table Size, that the mean prediction rate decreases as the table size increases.  This is to be expected as all branch predictors benefit from increased table size, and the perceptron is no exception.

### Confidence Prediction

We can see that he sensitivity of the perceptron is better than the other confidence predictors studied.  This means the perceptron was much better at covering the high confidence.  Most correct predictions of the branch predictor were correctly classified as high confidence, which is good.

However, the PVP is more important parameter to engineering.  Though many  correct predictions were identified as high confidence, many incorrect predictions were erroneously identified as high confidence as well.  The perceptron was very inefficient at predicting how confidence predictions.  We can its PVP value was roughly 25% less than the other confidence predictors.  This is just not an acceptable number to have as a confidence predictor.

The specificity of the perceptron was very poor.  This means that many incorrect predictions erroneously identified as high confidence, when they should have been low confidence.  Remember that a totally random assignment of confidence will give you a .5 specificity.  That’s 500% better than the perceptron.

The  perceptron did do significantly better than the other predictors in PVN.  However, taking specificity into account, the significance of the PVN of the perceptron becomes substantially weakened.  Its PVN was high simply because it could not correctly identify many incorrect predictions as low confidence.

# Conclusion

## Summary

In summary, the perceptron failed miserably at branch and confidence prediction.  Branch prediction rates were at best 6 and 5 percentage points less at 8 Kb and 64 Kb sizes respectively, than the conventional methods of two-level and hybrid prediction with equal hardware budgets.  When compared with equal entry sizes, the perceptron did better, showing a 5 and 1 percentage point improvement in misprediction rates.  However, the conventional schemes still had better misprediction rates.

The perceptron also was shown to be a poor confidence predictor.  PVP values were significantly lower, around 25 percentage points, than the agreement and branch clustering predictors.  The perceptron did show much better results than the other predictors in PVN statistics.  However, a specificity of 1% is not acceptable in a real world situation.

## Interpretation

### Branch prediction

The first thing we see about perceptron branch prediction is that it fails miserably when compared to two-level and hybrid branch prediction schemes.  The most obvious reason to this is there is a lot of destructive branch interference occurring.  Recall that at similar hardware budgets, the two-level schemes had 32x as many entries as the perceptron at the very least.  In a typical program, there are hundreds of millions of branches, and the more entries in the tables the less aliasing will occur.  The less aliasing that happens, the less destructive interference occurs, and predictor accuracies markedly improve.  We can see this by charting misprediction rates of the perceptron vs. table size.  As table size increases, misprediction rates decrease, which leads to better overall processor performance.

Based on the destructive interference I saw happening, I decided to test the perceptron with equal entry size table compared to the two-level.  To no surprise, its performance greatly improved.  However, it again could not beat the two-level scheme.  This simply shows that the perceptron is not well suited to branch prediction.

### Confidence prediction

In evaluating the confidence predictors, I have come up with the following formula to study the overall effectiveness of the varying predictors:

predictor effectiveness =  (sensitivity * PVP + specificity * PVN)  /  2.

Sensitivity * PVP represents the fraction of high-confidence predictions that were correctly identified.  Specificity * PVN represents the fraction of low-confidence predictions that were correctly identified.  These are normalized to obtain an overall score.  Simply put this is the fraction evaluates the effectiveness of the confidence predictor, in terms of how accurately it can identify a branch.  Here are the scores for  the varying predictors:

| Budget | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8Kb | 0.445 | 0.388 | 0.337 | 0.284 | 0.326 | 0.330 | 0.331 |
| 64Kb | 0.452 | 0.408 | 0.362 | 0.310 | 0.334 | 0.336 | 0.338 |

A general trend to note is the improvement of performance with larger hardware budgets.  This is to be expected, as larger hardware budgets produce better branch prediction accuracies.  Better branch prediction accuracies mean that there are less overall mispredictions.  Thus the confidence predictor can gain a better understanding of the branch’s behavior and be better able to make a confidence prediction.

As we can see from the data, the agreement predictor is by far the best predictor, performing 6 – 16% better at lower hardware budgets, and 5 – 14% better at larger hardware budgets when compared to other predictors.  The 8 Kb size even outperformed all 64 Kb entries of all the other predictors.  This shows it is a much better predictor than either branch clustering or the perceptron.

Another statistic to note is the trend between high thresholds for the branch clustering predictor and the perceptron predictor.  Overall, branch clustering performs better with a more aggressive (i.e. lower) confidence threshold, while the perceptron performs better with more conservative estimates.  This should not be that surprising, as their implementations are different.  Branch clustering is based on the accuracies of past branches, while the perceptron is based on the accuracy of the current branch.  This data shows that it makes more sense to make a more conservative estimate of high-confidence when dealing on a per branch basis, and more aggressive when dealing with groups of branches as a whole.

## Recommendations

Based on my experimental results, the perceptron is a vastly inferior to conventional methods of branch and confidence prediction.  Misprediction rates were far so inferior to that of the two-level and hybrid models that a significant decrease will be seen in processor performance if implemented in hardware.  The structure of a perceptron is very inefficient in that it needs many more bits (6-7 vs. 2) to store its weights.  Combined with the extra dimension in its array structure, the amount of bits per entry dramatically decreases the number of entries allowable in an equal hardware budget.  Hardware budgets aside, its performance was still inferior.  As a confidence predictor, it outperformed the agreement predictor and branch clustering confidence predictors.  A specificity of 1%, however, is entirely too low a value to warrant implementation in hardware.  Therefore I see no reason to continue pursuing further research into using perceptrons for either branch or confidence prediction.

# Bibliography

- P. S. Ahuja, K. Skadron, M. Martonosi, and D.W. Clark, Multipath Execution: Opportunities and Limits.  Proceedings of the 1998 International Conference on Supercomputing, Jul. 1998.

- T. Austin, The SimpleScalar Architectural Research Tool Set, Version 2.0. http://www.simplescalar.org, Feb. 2000.

- E. B. Baatz, Making Brain Waves.  CIO Magazine, Jan. 15, 1995.

- D. C. Burger and T. M. Austin, The SimpleScalar Tool Set, Version 2.0.  Computer Architecture News, 25(3), pages 13 – 25, Jun. 1997.

- P.-Y. Chang, E. Hao, and Y. N. Patt, Alternative implementations of hybrid

branch predictors.  Proceedings of the 28th Annual International Symposium on Microarchitecture, pages 252 – 257, Dec. 1995.

- Robert Fuller, Introduction to Neural-Fuzzy Systems.  Physica-Verlag, Heidelberg, Germany, 2000.

- D. Grunwald, A. Klauser, S. Manne and A. Pleszkun, Confidence estimation for speculation control.  Proceedings of the 25th Annual International Symposium on Computer Architecture, pages 122 – 131, Jun. 1998.

- D. Grunwald, D. Lindsay, and B. Zorn, Static methods in hybrid branch prediction. Proceedings of the 1998 International Conference on Parallel Architectures and Compilation Techniques, pages 222 – 229, Oct. 1998.

- J. L. Hennessy & D. A. Patterson, Computer Architecture: A Quantitative Approach, 2nd ed.  Morgan Kaufmann Publishers, San Francisco, 1996.

- E. Jacobsen, E. Rotenberg, and J. E. Smith, Assigning confidence to conditional branch predictions.  Proceedings of the 29th International Symposium on Microarchitecture, pages 142 – 152, Dec. 1996.

- D. A. Jimenez and C. Lin, Dynamic Branch Prediction with Perceptrons.  Proceedings of the 7th International Symposium on High-Performance Computer Architecture, Jan. 2001.

- N. K. Kasabov, Foundations of neural networks, fuzzy systems, and knowledge engineering.  MIT Press, Cambridge, MA, 1996.

- S. Manne, A. Klauser and D. Grunwald, Pipeline gating: speculation control for energy reduction,  Proceedings of the 25th annual international symposium on Computer architecture, pages 132 – 141, Jan. 1998

- J. Marciniak, Encyclopedia of Software Engineering, Vol. 1.  John Wiley & Sons, Inc., New York, 1992.

- S. McFarling, Combining branch predictors.  Tech. Note TN-36, DEC WRL,

Jun. 1993.

- S.-T. Pan, K. So, and J. T. Rahmeh, Improving the accuracy of dynamic branch prediction using branch correlation.  Proceedings of the Fifth International Conference on Architectural Support for Programming Languages and Operating Systems, pages 76 – 84, Oct. 1992.

- W. S. Sarle, Neural Network FAQ, part 1 of 7: Introduction.   http://www.informatik.uni-freiburg.de/~heinz/FAQ.html, May 2000.

- S. Shapiro, Encyclopedia of Artificial Intelligence, 2nd Edition, Vol. 2.  John Wiley & Sons, Inc., New York, 1992.

- K. Skadron, Thesis Technical Advisor. Assistant Professor, Dept. of Computer Science, University of Virginia.  http://www.cs.virginia.edu/~skadron/research.html, 2001.

- K. Skadron, Director, Laboratory for Computer Architecture at Virginia. http://lava.cs.virginia.edu, 2001.

- K. Skadron, P. S. Ahuja, M. Martonosi, and D. W. Clark, Branch Prediction, Instruction-Window Size, and Cache Size: Performance Tradeoffs and Simulation Techniques.  IEEE Transactions on Computers, 48(11):1260-81, Nov. 1999.

- K. Skadron, Characterizing and Removing Branch Mispredictions.  PhD thesis, Princeton Univ.; also Tech. Report TR-604-99; Jun. 1999.

- K. Skadron, M. Martonosi, and D.W. Clark, A Taxonomy of Branch Mispredictions, and Alloyed Prediction as a Robust Solution to Wrong-History Mispredictions.   Proceedings of the 2000 International Conference on Parallel Architectures and Compilation Techniques, pages 199-206, Oct. 2000.

- J. E. Smith, A study of branch prediction strategies.  Proceedings of the8th Annual International Symposium on Computer Architecture, pages 135 – 48, May 1981.

- T.-Y. Yeh and Y. N. Patt, Alternative implementations of two-level adaptive

branch prediction.  Proceedings of the 19th Annual International Symposium on Computer Architecture, pages 124 – 134, May 1992.

# Appendix A – Implementation

## A.1 Initialization of Data Structures

perceptron_create (

unsigned int weight_table_size,	 	/* level-1 table size */

unsigned int shift_width,	/* history register width */

{

int cnt;

### A.1.1 Weight Table

if (!weight_table_size || (weight_table_size & (weight_table_size –

~1))!= 0)

fatal("weight table size, `%d', must be non-zero and a power of /

/two", weight_table_size);

size = weight_table_size;

history_length = shift_width;

if (!(pred_dir->config.percept.table = calloc(weight_table_size,

~sizeof(int*))))

fatal("cannot allocate weight table storage");

for (cnt = 0; cnt < l1size; cnt++)

{

if (!(pred_dir->config.percept.table[cnt] =

/calloc(shift_width, sizeof(int))))

fatal("cannot allocate weight table storage");

}

### A.1.2 GBHR

if (!(pred_dir->config.percept.GBHR = calloc(shift_width,

~sizeof(int))))

fatal("cannot allocate GBHR storage");

### A.1.3 Threshold

threshold = 1.93 * history_length + 14;

}

## A.2 Prediction Calculation

int perceptron_calculation(

address branch_addr)		/* branch address */

{

int sum = 0;

int cnt = 0;

for (cnt = 0; cnt < history_length; cnt++)

{

/* Compute dot product */

sum += GBHR[cnt] * weight_table[branch_addr][cnt];

}

return sum;

}

## A.3 Direction Calculation

int percetron_predicted_direction(

address branch_addr)		/* branch address */

{

/* return 1 for a taken prediction, -1 for a not taken */

if (branch_prediction_calculation(branch_address) >= 0)

/* branch is predicted taken */

return 1;

else

/* branch is predicted not taken */

return –1;

}

## A.4 Updating Functions

void perceptron_update(

address branch_addr,		/* branch address */

int taken,	     		/* direction outcome of branch

1 if branch was taken

-1 if branch was not taken */

{

int cnt = 0;

### A.4.1 Weight Table Update

perceptron_output = bpred_calc(branch_addr);

if (perceptron_output < 0)

Perceptron_output = -1 * perceptron_output;

if ((pred_taken != taken) || (percept_output <= threshold))

for (cnt = 0; cnt < history_length; cnt++)

{

weight_table[branch_addr][cnt] += GBHR[cnt] * taken;

}

### A.4.2 GBHR Update

GBHR[0] = 1;

for (cnt = 1; cnt < history_length - 1; cnt++)

{

GBHR[cnt+1] = GBHR[cnt];

}

GBHR[1] = taken;

}

## A.4 Confidence Prediction

void confidence_prediction(

int taken,	     		/* direction outcome of branch

1 if branch was taken

-1 if branch was not taken */

int pred_taken)   		/* predicted direction of branch

1 if branch was predicted taken

-1 if branch was predicted not taken */

int bimod_prediction,	/* bimodal predicted direction */

int two_level_prediction,  /* two-level predicted direction */

int cluster_threshold,	/* threshold value for branch cluster high

confidence */

int perceptron_threshold	/* threshold value for perceptron high

confidence */

int perceptron_output)	/* value of perceptron calculation */

{

int i;

### A.5.1 Bimodal and Two-level Agreement

if (bimodal_prediction == two_level_prediction)

/* high confidence */

{

if (pred_taken == taken)

/* correct prediction */

++chc[0];

else

/*incorrect prediction */

++ihc[0];

}

else

/* low confidence */

{

if (pred_taken == taken)

/* correct prediction */

++clc[0];

else

/* incorrect prediction */

++ilc[0];

}

### A.5.2 Branch Misprediction Clustering

++last_misprediction; /*number of branches since last misprediction*/

if (last_misprediction > cluster_threshold)

/* high confidence */

{

if (pred_taken == taken)

/* correct prediction */

++chc[1];

else

/* incorrect prediction */

++ihc[1];

}

else

/* low confidence */

{

if (pred_taken == taken)

/* correct prediction */

++clc[1];

else

/* incorrect prediction */

++ilc[1];

}

### A.5.3 Perceptron

if (perceptron_output > perceptron_threshold)

/* high confidence */

{

if (pred_taken == taken)

/* correct prediction */

++chc[2];

else

++ihc[2];

}

else

/* low confidence */

{

if (pred_taken == taken)

/* correct prediction */

++clc[2];

else

/* incorrect prediction */

++ilc[2];

}

### A.5.4 Statistical Analysis

for(i = 0; i < 3; ++i)

{

if (chc[i] || clc[i])	/* check that denominator != 0 */

sens[i] = chc[i] / (chc[i] + clc[i]);

if (chc[i] || ihc[i])

pvp[i] = chc[i] / (chc[i] + ihc[i]);

if (ihc[i] + ilc[i])

spec[i] = ilc[i] / (ihc[i] + ilc[i]);

if (pred->clc[i] + pred->ilc[i])

pvn[i] = ilc[i] / (clc[i] + ilc[i]);

}

}

# Appendix B – Additional Data

## B.1 Misprediction Rates

| Predictor |  |  |  | Benchmark |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8Kb Budget | Compress | Gcc | Go | Perl | Radiosity | Gnuchess |  | Wolf | Avg |
| Perceptron - 32 | 0.146 | 0.278 | 0.341 | 0.253 | 0.193 | 0.257 |  | 0.213 | 0.240 |
| Perceptron - 64 | 0.134 | 0.233 | 0.308 | 0.191 | 0.185 | 0.221 |  | 0.196 | 0.210 |
| Perceptron - 128 | 0.132 | 0.194 | 0.284 | 0.147 | 0.157 | 0.199 |  | 0.176 | 0.184 |
| Perceptron - 4096, 62 | 0.132 | 0.102 | 0.201 | 0.085 | 0.148 | 0.169 |  | 0.132 | 0.138 |
| Perceptron - 4096, 5 | 0.130 | 0.101 | 0.199 | 0.087 | 0.158 | 0.164 |  | 0.117 | 0.137 |
| 2Level | 0.117 | 0.109 | 0.215 | 0.070 | 0.089 | 0.147 |  | 0.110 | 0.122 |
| Hybrid | 0.118 | 0.106 | 0.209 | 0.068 | 0.110 | 0.159 |  | 0.100 | 0.124 |
| 64Kb Budget |  |  |  |  |  |  |  |  |  |
| Perceptron - 256 | 0.131 | 0.170 | 0.259 | 0.108 | 0.148 | 0.181 |  | 0.172 | 0.167 |
| Perceptron - 512 | 0.131 | 0.142 | 0.236 | 0.108 | 0.147 | 0.175 |  | 0.154 | 0.156 |
| Perceptron - 1024 | 0.130 | 0.120 | 0.218 | 0.093 | 0.153 | 0.169 |  | 0.136 | 0.146 |
| Perceptron - 32768, 62 | 0.132 | 0.094 | 0.197 | 0.083 | 0.148 | 0.169 |  | 0.128 | 0.136 |
| Perceptron - 32768, 8 | 0.130 | 0.092 | 0.194 | 0.085 | 0.148 | 0.165 |  | 0.116 | 0.133 |
| 2Level | 0.102 | 0.071 | 0.162 | 0.037 | 0.064 | 0.096 |  | 0.107 | 0.091 |
| Hybrid | 0.103 | 0.068 | 0.165 | 0.041 | 0.075 | 0.114 |  | 0.088 | 0.093 |

## B.2 Confidence Prediction

|  |  |  | 8 Kb Hardware Budget |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compress | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 81.55% | 73.45% | 58.72% | 43.73% | 65.77% | 65.40% | 64.23% |
| IHC | 6.31% | 8.35% | 7.09% | 4.65% | 32.26% | 31.76% | 31.00% |
| CLC | 8.98% | 17.08% | 31.81% | 46.80% | 1.11% | 1.48% | 2.66% |
| ILC | 3.16% | 1.12% | 2.38% | 4.82% | 0.86% | 1.35% | 2.12% |
| Sens | 0.901 | 0.811 | 0.649 | 0.483 | 0.983 | 0.978 | 0.960 |
| PVP | 0.928 | 0.898 | 0.892 | 0.904 | 0.671 | 0.673 | 0.674 |
| Spec | 0.333 | 0.119 | 0.251 | 0.509 | 0.026 | 0.041 | 0.064 |
| PVN | 0.260 | 0.062 | 0.070 | 0.093 | 0.436 | 0.478 | 0.444 |
| Gcc | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 79.06% | 75.47% | 63.79% | 50.55% | 55.77% | 53.88% | 52.32% |
| IHC | 6.17% | 7.09% | 5.46% | 3.98% | 39.67% | 37.93% | 36.53% |
| CLC | 11.61% | 15.20% | 26.88% | 40.12% | 2.36% | 4.25% | 5.81% |
| ILC | 3.16% | 2.24% | 3.87% | 5.35% | 2.20% | 3.94% | 5.34% |
| Sens | 0.872 | 0.832 | 0.704 | 0.557 | 0.959 | 0.927 | 0.900 |
| PVP | 0.928 | 0.914 | 0.921 | 0.927 | 0.584 | 0.587 | 0.589 |
| Spec | 0.339 | 0.241 | 0.414 | 0.574 | 0.053 | 0.094 | 0.128 |
| PVN | 0.214 | 0.129 | 0.126 | 0.118 | 0.483 | 0.481 | 0.479 |

| Go | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHC | 67.08% | 57.15% | 40.26% | 24.69% | 60.17% | 59.29% | 58.45% |
| IHC | 10.80% | 11.27% | 7.60% | 4.25% | 37.10% | 36.07% | 35.29% |
| CLC | 15.43% | 25.36% | 42.25% | 57.82% | 1.34% | 2.22% | 3.06% |
| ILC | 6.69% | 6.22% | 9.89% | 13.24% | 1.39% | 2.41% | 3.20% |
| Sens | 0.813 | 0.693 | 0.488 | 0.299 | 0.978 | 0.964 | 0.950 |
| PVP | 0.861 | 0.835 | 0.841 | 0.853 | 0.619 | 0.622 | 0.624 |
| Spec | 0.382 | 0.356 | 0.565 | 0.757 | 0.036 | 0.063 | 0.083 |
| PVN | 0.302 | 0.197 | 0.190 | 0.186 | 0.509 | 0.521 | 0.511 |
| Perl | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 86.98% | 84.53% | 75.64% | 63.78% | 66.91% | 65.81% | 64.44% |
| IHC | 3.98% | 4.83% | 4.38% | 3.78% | 29.39% | 28.28% | 27.18% |
| CLC | 7.58% | 10.02% | 18.91% | 30.77% | 1.83% | 2.93% | 4.30% |
| ILC | 1.47% | 0.62% | 1.07% | 1.67% | 1.87% | 2.98% | 4.08% |
| Sens | 0.920 | 0.894 | 0.800 | 0.675 | 0.973 | 0.957 | 0.937 |
| PVP | 0.956 | 0.946 | 0.945 | 0.944 | 0.695 | 0.699 | 0.703 |
| Spec | 0.270 | 0.113 | 0.196 | 0.306 | 0.060 | 0.095 | 0.130 |
| PVN | 0.162 | 0.058 | 0.053 | 0.051 | 0.505 | 0.504 | 0.486 |
| Gnuchess | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 76.32% | 63.58% | 49.54% | 36.01% | 55.59% | 52.24% | 49.62% |
| IHC | 10.16% | 9.93% | 6.37% | 3.68% | 35.89% | 32.88% | 30.63% |
| CLC | 9.28% | 22.02% | 36.06% | 49.58% | 4.40% | 7.75% | 10.37% |
| ILC | 4.24% | 4.47% | 8.04% | 10.72% | 4.12% | 7.13% | 9.38% |
| Sens | 0.892 | 0.743 | 0.579 | 0.421 | 0.927 | 0.871 | 0.827 |
| PVP | 0.883 | 0.865 | 0.886 | 0.907 | 0.608 | 0.614 | 0.618 |
| Spec | 0.295 | 0.310 | 0.558 | 0.744 | 0.103 | 0.178 | 0.234 |
| PVN | 0.314 | 0.169 | 0.182 | 0.178 | 0.483 | 0.479 | 0.475 |
| Radiosity | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 80.36% | 85.20% | 77.07% | 66.58% | 66.91% | 65.81% | 64.44% |
| IHC | 3.60% | 4.32% | 3.95% | 3.33% | 29.39% | 28.28% | 27.18% |
| CLC | 14.10% | 9.27% | 17.40% | 27.88% | 1.83% | 2.93% | 4.30% |
| ILC | 1.93% | 1.22% | 1.58% | 2.20% | 1.87% | 2.98% | 4.08% |
| Sens | 0.851 | 0.902 | 0.816 | 0.705 | 0.973 | 0.957 | 0.937 |
| PVP | 0.957 | 0.952 | 0.951 | 0.952 | 0.695 | 0.699 | 0.703 |
| Spec | 0.349 | 0.220 | 0.286 | 0.398 | 0.060 | 0.095 | 0.130 |
| PVN | 0.120 | 0.116 | 0.083 | 0.073 | 0.505 | 0.504 | 0.486 |
| Wolf | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 83.32% | 78.82% | 69.82% | 59.87% | 60.40% | 57.63% | 55.42% |
| IHC | 5.45% | 5.65% | 4.25% | 3.05% | 29.95% | 27.81% | 26.23% |
| CLC | 7.94% | 12.45% | 21.45% | 31.40% | 5.67% | 8.44% | 10.66% |
| ILC | 3.28% | 3.08% | 4.48% | 5.68% | 3.97% | 6.11% | 7.69% |
| Sens | 0.913 | 0.864 | 0.765 | 0.656 | 0.914 | 0.872 | 0.839 |
| PVP | 0.939 | 0.933 | 0.943 | 0.952 | 0.669 | 0.675 | 0.679 |
| Spec | 0.376 | 0.353 | 0.513 | 0.651 | 0.117 | 0.180 | 0.227 |
| PVN | 0.292 | 0.198 | 0.173 | 0.153 | 0.412 | 0.420 | 0.419 |

|  |  |  | 64 Kb Hardware Budget |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compress | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 81.71% | 76.79% | 64.00% | 50.00% | 65.64% | 65.33% | 64.14% |
| IHC | 5.21% | 7.31% | 6.14% | 4.33% | 32.11% | 31.72% | 30.93% |
| CLC | 10.08% | 15.00% | 27.79% | 41.79% | 1.25% | 1.56% | 2.74% |
| ILC | 3.00% | 0.91% | 2.07% | 3.89% | 1.01% | 1.39% | 2.19% |
| Sens | 0.890 | 0.837 | 0.697 | 0.545 | 0.981 | 0.977 | 0.959 |
| PVP | 0.940 | 0.913 | 0.912 | 0.920 | 0.672 | 0.673 | 0.675 |
| Spec | 0.366 | 0.110 | 0.252 | 0.473 | 0.031 | 0.042 | 0.066 |
| PVN | 0.230 | 0.057 | 0.069 | 0.085 | 0.448 | 0.473 | 0.444 |
| Gcc | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 84.60% | 83.15% | 74.15% | 63.03% | 56.74% | 55.92% | 55.03% |
| IHC | 4.11% | 5.09% | 4.31% | 3.44% | 40.51% | 39.72% | 38.88% |
| CLC | 9.24% | 10.68% | 19.68% | 30.80% | 1.39% | 2.21% | 3.10% |
| ILC | 2.05% | 1.08% | 1.86% | 2.72% | 1.36% | 2.15% | 2.99% |
| Sens | 0.902 | 0.886 | 0.790 | 0.672 | 0.976 | 0.962 | 0.947 |
| PVP | 0.954 | 0.942 | 0.945 | 0.948 | 0.583 | 0.585 | 0.586 |
| Spec | 0.333 | 0.175 | 0.301 | 0.442 | 0.033 | 0.051 | 0.071 |
| PVN | 0.182 | 0.092 | 0.086 | 0.081 | 0.495 | 0.494 | 0.491 |
| Go | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 70.14% | 64.66% | 48.98% | 33.13% | 60.36% | 59.75% | 59.17% |
| IHC | 7.64% | 9.85% | 7.19% | 4.49% | 37.40% | 36.77% | 36.17% |
| CLC | 16.07% | 21.55% | 37.22% | 53.07% | 1.15% | 1.76% | 2.34% |
| ILC | 6.15% | 3.95% | 6.61% | 9.31% | 1.09% | 1.72% | 2.32% |
| Sens | 0.814 | 0.750 | 0.568 | 0.384 | 0.981 | 0.971 | 0.962 |
| PVP | 0.902 | 0.868 | 0.872 | 0.881 | 0.617 | 0.619 | 0.621 |
| Spec | 0.446 | 0.286 | 0.479 | 0.675 | 0.028 | 0.045 | 0.060 |
| PVN | 0.277 | 0.155 | 0.151 | 0.149 | 0.487 | 0.494 | 0.498 |

| Perl | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHC | 90.68% | 89.88% | 83.43% | 74.30% | 66.72% | 66.01% | 64.31% |
| IHC | 2.51% | 3.29% | 3.22% | 2.95% | 29.18% | 28.51% | 27.03% |
| CLC | 5.89% | 6.69% | 13.14% | 22.27% | 2.03% | 2.74% | 4.43% |
| ILC | 0.92% | 0.14% | 0.22% | 0.49% | 2.08% | 2.74% | 4.23% |
| Sens | 0.939 | 0.931 | 0.864 | 0.769 | 0.971 | 0.960 | 0.936 |
| PVP | 0.973 | 0.965 | 0.963 | 0.962 | 0.696 | 0.698 | 0.704 |
| Spec | 0.268 | 0.042 | 0.063 | 0.142 | 0.066 | 0.088 | 0.135 |
| PVN | 0.135 | 0.021 | 0.016 | 0.021 | 0.506 | 0.500 | 0.488 |
| Gnuchess | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 77.31% | 72.49% | 59.41% | 45.01% | 68.73% | 67.46% | 66.19% |
| IHC | 6.37% | 8.08% | 6.15% | 4.18% | 27.41% | 26.21% | 25.16% |
| CLC | 12.40% | 17.22% | 30.30% | 44.70% | 2.01% | 3.28% | 4.55% |
| ILC | 3.92% | 2.21% | 4.14% | 6.10% | 1.85% | 3.05% | 4.10% |
| Sens | 0.862 | 0.808 | 0.662 | 0.502 | 0.972 | 0.954 | 0.936 |
| PVP | 0.924 | 0.900 | 0.906 | 0.915 | 0.715 | 0.720 | 0.725 |
| Spec | 0.381 | 0.215 | 0.403 | 0.593 | 0.063 | 0.104 | 0.140 |
| PVN | 0.240 | 0.114 | 0.120 | 0.120 | 0.480 | 0.482 | 0.474 |

| Radiosity | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHC | 80.36% | 85.20% | 77.07% | 66.58% | 66.72% | 66.01% | 64.31% |
| IHC | 3.60% | 4.32% | 3.95% | 3.33% | 29.18% | 28.51% | 27.03% |
| CLC | 14.10% | 9.27% | 17.40% | 27.88% | 2.03% | 2.74% | 4.43% |
| ILC | 1.93% | 1.22% | 1.58% | 2.20% | 2.08% | 2.74% | 4.23% |
| Sens | 0.851 | 0.902 | 0.816 | 0.705 | 0.971 | 0.960 | 0.936 |
| PVP | 0.957 | 0.952 | 0.951 | 0.952 | 0.696 | 0.698 | 0.704 |
| Spec | 0.349 | 0.220 | 0.286 | 0.398 | 0.066 | 0.088 | 0.135 |
| PVN | 0.120 | 0.116 | 0.083 | 0.073 | 0.506 | 0.500 | 0.488 |
| Wolf | Agree | Cluster 2 | Cluster 4 | Cluster 7 | Percept 1/3 | Percept 1/2 | Percept 2/3 |
| CHC | 83.41% | 81.77% | 74.17% | 65.73% | 56.80% | 55.58% | 52.18% |
| IHC | 4.47% | 4.69% | 3.62% | 2.57% | 28.15% | 27.30% | 25.01% |
| CLC | 8.95% | 10.58% | 18.19% | 26.63% | 9.28% | 10.50% | 13.90% |
| ILC | 3.17% | 2.95% | 4.02% | 5.07% | 5.77% | 6.62% | 8.91% |
| Sens | 0.903 | 0.885 | 0.803 | 0.712 | 0.860 | 0.841 | 0.790 |
| PVP | 0.949 | 0.946 | 0.954 | 0.962 | 0.669 | 0.671 | 0.676 |
| Spec | 0.415 | 0.386 | 0.527 | 0.663 | 0.170 | 0.195 | 0.263 |
| PVN | 0.262 | 0.218 | 0.181 | 0.160 | 0.384 | 0.387 | 0.391 |