Preface

The principal investigator in this project, Adrian V. Lanning,  is a fourth year undergraduate in the Department of Electrical Engineering at the University of Virginia.  Mr. Lanning will graduate with a concentration in Digital Systems and a minor in Computer Science.  This track is known in the School of Engineering and Applied Science as the Computer Engineering curriculum.

Mr. Lanning has taken many classes relevant to this project including EE335: “Microcomputer Architecture,” EE407: “Fault Tolerant Computing,” EE435: “Computer Organization and Design,” and especially CS551: “Advanced Topics in Computer Architecture: A Microprocessor Survey.”  It was through this class that Mr. Lanning first met his Technical Advisor, Dr. Kevin Skadron.

Mr. Lanning’s interests lie in the field of embedded computing and microarchitecture design.  He is passionate about hardware design and enjoys interfacing software programs with hardware devices he has designed and/or implemented.  It is hoped that through this project, Mr. Lanning may gain a better insight into the design and simulation of today’s computer hardware.

Preface…..………………….………………………………………………………...ii

Table of Figures……..………………………………………………………....…iv

Glossary of terms...……………………………………………………………...v

ABSTRACT.……………………………………………………………………………..vi

Chapter 1. Introduction	1

1-1. Pipelined Processors	2

1-2. Branch Prediction	4

Bimodal Predictors	5

Two-Level Predictors	6

Hybrid Predictors	8

1-3. Rationale	10

Per-Branch Needed-History Tracking	10

Dynamic vs. Static Predictors	12

Ideal vs. Realistic Predictor Configurations	13

1-4. Overview of Contents	13

Chapter 2. Characterizing Wrong-History	14

2-1. Description of Process	14

SimpleScalar Instruction Set Simulator	14

SPECint95 Benchmark Programs	15

2-2. Description of Equipment	15

2-3. Predictor Configurations	16

2-4. Simulation Configurations	17

Chapter 3. Results and Discussion	18

3-1. Scope of Testing	18

3-2. Dynamic vs. Static Results	19

3-3. Per-Branch Needed-History Results	20

Chapter 4. Conclusions	22

4-1. Summary	22

Static vs. Dynamic Summary	22

Wrong-History Summary	22

BHT Conflicts Summary	23

4-2. Interpretation	24

4-3. Recommendations for Future Work	24

4-4. Final Word	25

Works Cited….……………………………………………………………………..32

Table of Figures

Figure 1.    Instruction Pipeline of the Intel Pentium III [Drawn by author].	3

Figure 2.    Bimodal Predictor Structure [8].	5

Figure 3.    Local History Predictor Structure [8].	7

Figure 4.    Global History Predictor Structure [8].	8

Figure 5.    Hybrid Predictor Structure [8].	9

Figure 6.    Go: Per-Branch Data for Ideal Configuration [Drawn by author].	1

Figure 7.    Go: Per-Branch Data for Realistic Configuration [Drawn by author].	1

Figure 8.    M88ksim:  Per-Branch Data for Ideal Configuration [Drawn by author].	2

Figure 9.    M88ksim: 	Per-Branch Data for Realistic Configuration [Drawn by author].	2

Figure 10.  Gcc: Per-Branch Data for Ideal Configuration [Drawn by author].	4

Figure 11.  Gcc: Per-Branch Data for Realistic Configuration [Drawn by author].	4

Figure 12.  Compress: Per-Branch Data for Ideal Configuration [Drawn by author].	5

Figure 13.  Compress: Per-Branch Data for Realistic Configuration [Drawn by author].	5

Figure 14.  Xlisp: Per-Branch Data for Ideal Configuration [Drawn by author].	6

Figure 15.  Xlisp: Per-Branch Data for Realistic Configuration [Drawn by author].	6

Figure 16.  Ijpeg: Per-Branch Data for Ideal Configuration [Drawn by author].	7

Figure 17.  Ijpeg: Per-Branch Data for Realistic Configuration [Drawn by author].	7

Figure 18.  Perl: Per-Branch Data for Ideal Configuration [Drawn by author].	8

Figure 19.  Perl: Per-Branch Data for Realistic Configuration [Drawn by author].	8

Figure 20.  Percentage of time Global (GAs), Ideal Local (PAp), and Realistic Local (PAs) predicted correctly. [Drawn by Author]	23

Glossary of Terms

Branch - A change in the control flow of a program.

Bimodal branch predictors - A simple branch predictor that tracks the taken/not-taken 	history of each branch.

Conflicts - Occur in predictor hardware when several branches or branch patterns map to 	the same table entry, thereby interfering with each other and possibly polluting 	the prediction.

Dynamic hybrid predictors – A hybrid predictor that dynamically selects between its 	internal predictors during program execution.

Local branch predictors - A type of two-level configuration that considers each branch 	independently and exploits individual repeating branch behavior.

Global branch predictors - A type of two-level configuration that combines the history 	of all recent branches when making a prediction.  This exploits inter-branch 	correlation.

Hybrid branch predictors - A predictor that contains two or more other predictors and 	chooses which prediction to use based on some kind of selection mechanism.

Needed-history type - used to refer to the type of branch predictor that performs best for 	a given branch.

Program counter - a special register where the processor keeps the memory address of 	the current instruction.

Static hybrid predictors – A hybrid predictor that assigns each branch to one of its 	internal predictors only once.

Wrong-history mispredictions – A misprediction in a branch predictor caused by the 	predictor using the wrong type of history.  For example, a hybrid predictor using 	its local predictor and predicting incorrectly when its global predictor would have 	predicted correctly.

Abstract

Although many advances have been made in the field of branch prediction, current research does not address two important problem areas: accurately dealing with frequently changing branch history types and quantitatively comparing static to dynamic hybrid predictor performance.  This report shows that most branches do change needed branch predictor types.  This report then goes on to show that these changes incur a significant performance decrease in static hybrid predictors.

Branch prediction research focuses on improving the performance of pipelined microprocessors by accurately predicting ahead of time whether or not a change in control flow will occur.  Changes in control flow (or branches) affect processor performance because many processor cycles must be wasted flushing the pipeline and reading in the correct instructions when programs do not behave as the processor expects them to.

Traditional dynamic hybrid predictors contain multiple branch predictors which track different branch history patterns and dynamically select between the two during program execution.  Static hybrid predictors also contain multiple branch predictors but assign each branch to a specific predictor at run time.  Statically assigning branches to predictors would decrease the selector hardware needed in a dynamically assigning hybrid predictor yet would decrease overall predictor accuracy if many of the individual branches changed the type of predictor they perform best with over time.  When this changing of needed-predictor (or history) types causes the predictor to make a bad prediction, a wrong-history misprediction is said to have occurred.

In order to determine the severity of wrong-history mispredictions in common programs, selected programs from the SPECint95 benchmark suite were simulated on an instruction set simulator known as SimpleScalar.  This report shows that most of the individual branches in the SPECint95 benchmark programs do alter needed-predictor types, causing wrong-history mispredictions to occur.  This report then goes on to compare the accuracy of the static predictor with that of a dynamic hybrid predictor.  Through this comparison, it is shown that wrong-history mispredictions account for a significant performance decrease in static hybrid predictors.

# Introduction

Branch prediction research focuses on improving the performance of pipelined microprocessors by accurately predicting ahead of time whether or not a change in control flow will occur.  Changes in control flow (or branches) affect processor performance because many processor cycles must be wasted flushing the pipeline and reading in the correct instructions when programs do not behave as the processor expects them to.

Traditional dynamic hybrid predictors contain multiple branch predictors which track different branch history patterns and dynamically select between the two during program execution [8].  Static hybrid predictors also contain multiple branch predictors but assign each branch to a specific predictor at run time.  Statically assigning branches to predictors would decrease the selector hardware needed in a dynamically assigning hybrid predictor yet would decrease overall predictor accuracy if many of the individual branches changed the type of predictor they perform best with over time.  When this changing of needed-predictor (or history) types causes the predictor to make a bad prediction, a wrong-history misprediction is said to have occurred.

This report shows that many programs contain branches which alter needed-history types, thereby reducing the overall accuracy of predictors which are not capable of adapting to changing branch behavior such as the static hybrid predictor – resulting in an overall performance decrease of the processor.  Section 1 follows with a description of modern processor architectures and helps describe why branch predictors are necessary.

## Pipelined Processors

The need for branch prediction arises from the use of pipelining in modern microarchitectures [5].  The goal of pipelining is to maximize utilization of all the independent components of a processor at once.  One useful analogy for visualizing the instruction flow in modern processors is the manufacturing of an automobile on an assembly line.

When a car is being constructed, the frame moves slowly down a conveyor belt while more pieces are attached in an ongoing process.  More importantly, once one car frame passes a certain stage in the construction, another frame may be brought in and worked on.  This type of parallel construction routine helps maximize the total throughput of the automobile plant by utilizing as much of the machinery as possible at the same time.  In a modern manufacturing plant, car frames may be pieced together, at the same time that the engine is put in more completed units, while the nearly-finished cars are being painted.

Similarly, a computer pipeline may be thought of as analogous to an automobile conveyor belt.  In a pipeline, however, program instructions replace the cars as the items being processed.  As the instruction moves down the pipeline, more and more pieces of its execution become complete.  The key to achieving parallelism, though, is that once an instruction has finished a stage in the pipeline, a subsequent instruction may enter that stage.  In a modern processor, instructions may be fetched from memory, while previous instructions are being decoded, and while the nearly-finished instructions are being executed [5].  As an example, Figure 1 displays the pipeline of the Intel Pentium III®.

Branch Prediction	       Fetch	          Decode                           Dispatch	       Execution

Figure 1. Instruction Pipeline of the Intel Pentium III [Drawn by Author].  Note the many steps between the fetch of the instruction from memory and the actual execution of an instruction. Figure 1 is drawn based on Pentium® processor development manuals reviewed in [11].

Since the goal of pipelining is to utilize the hardware to the fullest possible extent all the time, it is necessary to make sure that each stage of the pipeline contains an instruction as often as possible.  If there are no changes in program control flow, then the solution is simple, just make sure that instructions are read from memory quickly enough to keep all the stages full all the time.  However, when branches cause the program to behave in ways that the processor does not expect, the solution becomes much more complicated.  A branch is a change in the control flow of a program which breaks sequentiality.

Imagine that a branch instruction has moved through the fetch and decode stages and is now being executed.  This execution stage is the first time that the processor knows whether or not the branch will be taken.  In general the result of this decision is based on a compare between two other data elements (for example, IF X > Y THEN…).  The problem arises because until this comparison occurs, the processor does not know the next correct instruction to execute.  The stages prior to the execution cycle have already begun speculatively processing instructions that follow the branch, yet if the branch is taken, these are not the correct instructions.  Therefore, all the stages before the execution cycle must be flushed and instruction fetch must precede from the target location of the taken branch.

This flushing of the pipeline wastes many cycles of execution time, thereby decreasing the performance of the processor.  In an effort to save these wasted cycles, processor designers try to predict the direction of each branch instruction before the next instruction is fetched from memory [5: 200].  If the prediction is correct, the next instruction after the branch executes will be the correct instruction to execute next.  If the prediction is incorrect, however, the pipeline must be flushed, and the correct instruction read into the pipeline.  This incorrect prediction is known as a misprediction.

## Branch Prediction

Modern branch prediction techniques have evolved from simple pipeline stalls in which instructions following the branch instruction are delayed until the target is known to advanced history tables and dynamic selectors [5: 198].  The rationale for using precious silicon area for a fairly complex branch predictor comes directly from the performance benefits gained.  As Skadron et. al. point out, each misprediction costs, on average, 10 to 20 cycles of delay depending on the specific processor architecture [12].  They further go on to show that even using a predictor twice the size of that found in the Alpha 21264 results in a 7 percent misprediction rate, and a 20 percent performance penalty.  In fact, Jouppi and Ranganathan argue that branch prediction will be the most important bottleneck for processor performance by 2010 [6].  To better understand how modern branch predictors function, we will now look at several of the predictor types that have been proposed to date.

### Bimodal Predictors

One of the simplest branch predictors which tracks the behavior of individual branches is the bimodal predictor.  Bimodal branch predictors take advantage of the fact that a branch can either be taken or not taken.  This bimodal distribution of branch behavior allows branch predictor designers to represent a given branch occurrence with a single bit.  Figure 2 shows one of the simplest implementations of a bimodal predictor [8].

Counts

PC

Taken			  Predict Taken

Figure 2. Bimodal Predictor Structure [8].

The figure shows a table of 2-bit counters, each indexed by the low order address bits of the program counter.  For each taken branch, the appropriate counter is incremented, whereas for each not-taken branch, the appropriate counter is decremented.  In addition, due to the 2-bit size restriction, each counter is not decremented past zero, nor incremented past three.  The most significant bit of the counter is used for the prediction, 1 being taken, 0 being not-taken.  In this manner, branches which are repeatedly taken will be predicted accurately as well as branches which are repeatedly not-taken.

The 2-bit counter size cannot change prediction instantly, requiring 1-2 mispredictions of the same type before changing its prediction.  This has the added benefit of tolerating one unusual branch direction (resulting in one misprediction) and predicting the usual direction for subsequent branches.  This type of predictor works very well when the branch is repeatedly taken or not-taken.  Bimodal predictors, however, can not accurately predict branches that have a repeating pattern of taken/not-taken.

### Two-Level Predictors

Yeh and Patt recognized that using two levels of tables instead of the bimodal predictor’s one would allow predictors to recognize repetitive patterns [15].  Furthermore, they realized that by changing the configuration of the two history tables, different types of history patterns could be tracked.  In [15] Yeh and Patt suggest two types of configurations which performed well for a variety of programs.

The first type of configuration suggested, designated a local predictor, attempts to base its prediction on the history pattern specific (or local) to the current branch.  Figure 3 shows the general design of the local predictor.  As shown, the branch address is used to index the first history table (called a Branch History Table or BHT).  The value stored in the BHT represents the “direction taken by the most recent n branches whose addresses map to this entry, where n is the length of the entry in bits.” [8]

History	     Counts

Taken 			          		           Predict Taken

BHT

PHT

PC

Figure 3. Local History Predictor Structure [8].

The pattern stored in the BHT is then used as an address to index into the array of 2-bit counters, similar to the bimodal predictor structure.  Using the last n references to the specific branch as stored in the BHT allows the local predictor to use a different 2-bit counter, and thus a potentially different prediction, based on the pattern of the branch history rather than the address of the branch as in the bimodal predictor [15].

Rather than look at the pattern of only the current branch as in the local predictor, the second type of two-level configuration that Yeh and Patt proposed in [15] uses the pattern of the most recent m branches to make a prediction.  This type of configuration, known as the global configuration, uses only a single entry for the BHT.  This entry (m bits in length) holds the taken/not-taken history of the last m branches in the program and is used to index into the PHT.  Figure 4 illustrates the general design of the global predictor.

Counts

Taken   	              Predict Taken

PHT

Global History

BHT

Taken

Figure 4. Global History Predictor Structure [8].

Global branch prediction takes advantage of the fact that the direction taken by the current branch may depend strongly on the direction of other branches [15].  For example, two subsequent IF statements would demonstrate this type of behavior since the second IF statement will not even be executed if the first is not-taken.

Two-level branch predictors provide an accurate way to recognize when a branch behaves in a certain pattern.  However, many branches change patterns throughout their life.  Given that the different branch predictors discussed have different advantages, the next question may be whether the advantages of both may be combined to form a new type of predictor.

### Hybrid Predictors

One of the most influential schemes to come of late was suggested by Scott McFarling and combines the local and global history predictors into one hybrid predictor [8].  The hybrid predictor monitors which type of history predictor performs better for a given branch, and uses one of a variety of selection mechanisms to choose between them [2].

McFarling proposes using a bimodal structure as the selector with the 2-bit array of counters updated with the prediction accuracy of the two predictors used instead of whether the branch was taken or not-taken.  For example, assume a 1 from the bimodal predictor means use predictor 1.  Then, if predictor 1 is correct while predictor 2 is incorrect, the counter should be incremented.  If predictor 2 is correct while predictor 1 is incorrect, the counter should be decremented.  If both predictor 1 and predictor 2 are correct or incorrect, then no action needs to be taken.  This behavior is achieved by subtracting the correctness of predictor 2 from the correctness of predictor 1.  Figure 5 shows the general configuration of the hybrid predictor.

PC

Counts

P1

P2

P1c-P2c		      useP1

Figure 5. Hybrid Predictor Structure [8].

This section has described the most common types of predictors used in modern branch predictors.  The simulations performed in this report compare the prediction accuracy of a static and dynamic hybrid branch predictor.  Each hybrid predictor contains one local and one global branch predictor.

## Rationale

The main goal of this project is to characterize the severity of wrong-history mispredictions.  In order to determine whether wrong-history mispredictions incur a significant performance decrease in static hybrid predictors, it is first necessary to determine whether individual branches do change their needed-predictor types over the course of program execution.  If individual branches are shown to alter needed-predictor types then it is possible to use this data to compare the performance between static and dynamic hybrid predictors.  It is also possible to test the effect that conflicts in the internal predictor hardware have on predictor performance by comparing an idealistic configuration of the predictors where internal conflicts do not occur with a more realistic configuration.  This section of Chapter 1 expounds on the goals behind these tests while Chapter 2 describes how they are conducted.  The results of these tests are presented in Chapter 3.

### Per-Branch Needed-History Tracking

Current research focuses primarily on performance losses that arise due to resource conflicts in the branch predictor hardware [9] [13].  Yet some recent research suggests that conflicts may not be as important a cause of error as wrong-history mispredictions.  In one such example, Skadron et al. [12] shows that conflicts only account for 15-20 percent of mispredictions in global-history predictors while another type of misprediction, wrong-history misprediction, accounts for 35-50 percent of the mispredictions.

Wrong-history mispredictions occur when a branch is behaving in one manner while the branch predictor tracks a different kind of behavior.  Local branch predictors consider each branch independently while global branch predictors combine the history of all recent branches in making a prediction.  In addition to most programs having some branches that need local predictor types while others need global predictor types, individual branches often change orientation between the two as well: sometimes needing local history, sometimes needing global history.  A hybrid predictor with a perfect selector, or meta-predictor, would account for this type of misprediction given that every type of predictor that a given branch needed was included as a possible selection choice.  However, in practice, meta-predictors do not always choose correctly and predictor types are usually limited in number, thereby allowing this type of misprediction to continue.

In order to characterize the severity of wrong-history mispredictions, it is necessary to understand how a given branch behaves over the course of program execution.  Therefore, this project tracks the needed-history types for branches in several SPECint95 benchmark programs as well as the number of times that each branch switched needed-history types.

In general, this project provides useful data to better describe the behavior of branches by simulating SPECint95 benchmark programs [14] on a modified microarchitecture simulator: SimpleScalar 3.0’s “sim-bpred” [1].  The modified version of sim-bpred.c sets up two branch predictors, one local and one global.  The SPECint95 benchmark programs are executed using these branch predictors and performance and needed history types are recorded on a per-branch basis.

### Dynamic vs. Static Predictors

Much of the research in the field of hybrid branch predictors has been on dynamically selecting between the two predictor types [3][7][10].  Dynamic selection occurs each time that the branch predictor is referenced.  However, some researchers suggest that using a static selection algorithm based on compiler hints reduces the necessary hardware size and may be equally accurate [4].  Static selection occurs once, with each branch getting assigned to one predictor or the other.  This raises the broader question of where the selection should occur: in the hardware, or in the software.  Designers of static predictors would seem to prefer the compiler to handle the selection, while designers of dynamic predictors would seem to favor the hardware.

Training data is used to configure the static predictor so that it will give the best prediction results over the widest range of programs.  The data generated by this project could be especially useful in the design of static hybrid predictors.  These predictors choose which type of history a branch will require based on hints from the compiler which are included in the branch instruction itself.  To make these hints, the compiler uses data gathered from profiling, a technique where a program is run repeatedly and monitored, then re-compiled, taking into account the new characteristic data.  Tracking the frequency of behavior switches and how long a branch required one type before switching to another would aid in determining what data to monitor during the profiling.

The per-branch data obtained in this project show the relative accuracy of running the benchmark programs on a static predictor versus running the benchmark programs on a dynamic predictor.

### Ideal vs. Realistic Predictor Configurations

One last goal of this project is to illustrate the performance difference when using an ideal predictor versus a more realistic predictor.  The ideal case implies very large predictors where conflicts in the predictor hardware do not occur and the more realistic case implies smaller predictor areas where conflicts do occur.  Conflicts occur in predictor hardware when several branches or branch patterns map to the same table entry, thereby interfering with each other and possibly polluting the prediction.

## Overview of Contents

Chapter 2 includes a description of the process used to obtain the project data as well as a description of the equipment used, and the predictor and simulator configurations used.  Chapter 3 then goes on to present the results of the tests, discussing each test in turn.  Chapter 4 concludes this report with a summary of the results, interpretations, recommendations for future work, and a final word on the impact of this project.

# Characterizing Wrong-History

This chapter describes the tools and methods used to achieve the goals outlined in Chapter 1.  Section 1 describes the instruction set simulator used to simulate the different branch predictors as well as the benchmark programs that were used to test those predictors.  Section 2 describes the computer systems the simulations were run on.  Section 3 describes the configurations of the predictors used while Section 4 concludes Chapter 2 with a description of the simulation configurations used.

## Description of Process

### SimpleScalar Instruction Set Simulator

To find out whether wrong-history mispredictions play a significant role in branch predictor performance, simulations were carried out on a modified version of SimpleScalar 3.0’s sim-bpred simulator [1].  Two series of simulations were performed, the first using “ideal” local predictor and a global predictor configurations and the second using more realistic conditions.  To get the best comparison for the given predictor size, configurations were chosen based on best overall performance for the entire SPEC95 benchmark suite as determined by Skadron et al. [12].

The modifications to sim-bpred included creating two branch predictors, one local, one global and recording certain statistics not normally saved by the original version.  Both predictors were referenced when a branch instruction was executed and the corresponding hit/miss statistics were recorded.  Only those branches who were predicted correctly by one predictor but not both were recorded.  Also, the data shows any time a branch changed from being correctly predicted by local to correctly predicted by global (and vice versa).

### SPECint95 Benchmark Programs

This project ran selected SPECint95 benchmarks on its simulator [14].  All benchmarks were compiled for SimpleScalar’s portable ISA (PISA) using gcc version 2.6.3 at maximum optimization.  Table 1 summarizes the benchmarks' characteristics (static sites as reported by Skadron et al. [12]).  All are compiled using gcc –03 –funroll-loops for the SimpleScalar PISA.

TABLE 1.

Benchmark Characteristics.

|  | Input | Conditional branch |  |
| --- | --- | --- | --- |
|  |  | static sites | dynamic refs |
| go | 9stone21 | 4,327 | 455 M |
| m88ksim | ctl | 231 | 110 M |
| gcc (cc1) | cccp.I | 14,245 | 190 M |
| compress | bigtest | 205 | 202 M |
| xlisp | 9queens | 271 | 154 M |
| ijpeg | penguin.ppm | 657 | 50 M |
| perl | scrabbl | 352 | 268 M |

## Description of Equipment

Simulations were run on the compute servers of the Department of Computer Science of the University of Virginia.  These compute servers use multiple Sun UltraSparc I and UltraSparc II processors with various amounts of memory for each.  Differences between the UltraSparc I and UltraSparc II architectures should not affect the results of this project since both use the same “endian-ness” and the same instruction set simulator was used for both.

## Predictor Configurations

To illustrate the intrinsic behavior of branches in the testbench programs without contamination by conflicts within the predictor hardware itself, the first series of simulations used an “ideal” configuration for the local predictor.  These conflicts arise when independent branches map to the same predictor entry.  The configuration used in these simulations has a first-level Branch History Table (BHT) of 512k entries in order to represent an interference-free BHT.  The second series of simulations were conducted using a more realistic BHT configuration of 1k entries.  The configurations for the two series of simulations appear in Table 2 and 3.  In both cases, a 4-way set-associative Branch Target Buffer(BTB) was used.

TABLE 2.

Predictor configurations used for the “ideal” predictors.

|  | index | BHT | PHT |
| --- | --- | --- | --- |
| Global | 7g, 7a | 1 entry | 16K entries |
| Local | 13p, 0a | 512K entries | 8K entries |

TABLE 3.

Predictor configurations used for the realistic predictors.

|  | index | BHT | PHT |
| --- | --- | --- | --- |
| Global | 7g, 7a | 1 entry | 16K entries |
| Local | 13p, 0a | 1K entries | 8K entries |

## Simulation Configurations

Table 4 illustrates the simulation configurations used in this project.  Programs were run until the number of instructions executed exceeded those in Table 4.  This was done to cut down on the total simulation times involved.  The number of instructions fast-forwarded refers to the number of instructions that were executed before the data started being collected.  For example, the program “go” was run for four billion instructions but only the last 100 million instructions were used in the data gathering.  Fast-forwarding keeps the results free of the influence of the behavior of the program during its start-up sequences.  This is beneficial because start-up behavior may not be characteristic of the most normal state of execution behavior.

TABLE 4.

Simulation Configurations.

| Benchmark | Number of Instructions Executed | Number of Instructions Fast-Forwarded |
| --- | --- | --- |
| go | 4,000,000,000 | 3,900,000,000 |
| m88ksim | 1,000,000,000 | 950,000,000 |
| cc1 | 1,000,000,000 | 900,000,000 |
| compress | 1,700,000,000 | 1,600,000,000 |
| xlisp | 1,000,000,000 | 900,000,000 |
| ijpeg | 873,000,000 | 823,000,000 |
| perl | 2,000,000,000 | 1,950,000,000 |

# Results and Discussion

Chapter 3 includes the results of the tests described in Chapters 1 and 2 as well as a general discussion of the more interesting data obtained.  Section 1 describes the scope of the overall programs represented by the data gathered.  Section 2 describes the performance results between the dynamic and static predictor simulations while Section 3 describes the results of the needed-history tests.

## Scope of Testing

In order to cut down on the number of total branches processed, only those branches which execute over 100,000 times are included in the results shown.  This limit also allows us to focus on only those branches that make up the bulk of the control flow execution of the benchmarks.  Table 5 displays the percentages of the total number of dynamic references that this 100,000 time limit represents for each benchmark.

TABLE 5

Percentage of total branch execution represented by 100,000 time limit.

|  | Total Dynamic References | Over 100,000 Only – Dynamic References | Percentage of Total Dynamic References Represented |
| --- | --- | --- | --- |
| Go | 454,561,809 | 403,893,374 | 88.85% |
| M88ksim | 110,481,426 | 103,190,306 | 93.40% |
| Gcc | 190,019,613 | 78,965,138 | 41.56% |
| Compress | 202,018,740 | 201,913,690 | 99.95% |
| Xlisp | 154,224,797 | 152,170,995 | 98.67% |
| Ijpeg | 49,620,517 | 46,379,100 | 93.47% |
| Perl | 267,666,267 | 260,631,712 | 97.37% |

## Dynamic vs. Static Results

Table 6 shows an estimation of the relative performance of a dynamic hybrid predictor with a perfect selector versus a static hybrid predictor.  This performance percentage was obtained by dividing half of the average number of changes per benchmark program by the sum of the average number of global-only hits plus the average number of local-only hits.

Since a change represents a difference in needed-history type from one dynamic reference to another, a static predictor will mispredict roughly half as many times as the average number of changes per branch.  Consider, for example, a branch which alternates between needing local and global.  Assume that that branch was referenced 50 times.  The number of changes is therefore 49.  A perfectly selecting dynamic hybrid predictor will predict correctly all four times.  A statically selecting hybrid predictor, however, will predict correctly for only 25 times, or roughly half of the number of times changed.  Therefore, comparing the half of the average number of changes per branch with the average times that that branch needed only one type of history or the other results in an estimation of the performance benefit of dynamic predictors over static predictors.

TABLE 6

Estimation of Performance Difference Between Dynamic and Static Predictor Types

| Realistic Configuration | Times Needed Only Global + Times Needed Only Local | Half the Average Number of Changes per Branch | Relative Performance Increase of Dynamic to Static |
| --- | --- | --- | --- |
| Go | 184,481 | 30,130 | 16.33% |
| M88ksim | 91,055 | 5,930 | 6.50% |
| Gcc | 45,978 | 5,571 | 12.12% |
| Compress | 1,334,968 | 278,471 | 20.86% |
| Xlisp | 217,313 | 13,064 | 6.01% |
| Ijpeg | 124,572 | 26,881 | 21.58% |
| perl | 126,049 | 8,949 | 7.10% |

Table 6 illustrates that in all cases, dynamic branch prediction is more accurate than static branch prediction.  Even for Xlisp which had the smallest performance benefit, the increase was still over 6 percent.  As stated before, for some processors, a misprediction rate of only 7 percent resulted in a 20 percent performance loss [12].  Two, ijpeg and compress, had gains of more than 20 percent!

## Per-Branch Needed-History Results

Figures 6 through 19 in Appendix A are plots of the needed history types per static site for each benchmark programs.  The first plot in the group shows the history type needed for static sites using the ideal BHT while the second plot in each group shows that needed while using the realistic BHT.  For all plots, the number of times that the branch predicted correctly with local only is shown on the Y-axis and the number of times that the branch predicted correctly with global only is shown on the X-axis.  Below each plot are mean and standard deviation calculations for the number of local types needed, the number of global types needed, and the number of changes between the two.

Figures 6 through 19 in Appendix A show that a large majority of the branches executed lie in between the major axes, indicating that both local and global history types are needed.  Summarizing the average and standard deviation values for the realistic configuration of each of the benchmarks results in Table 7.

TABLE 7

Summary of Benchmark Results

| REAL | Times Needed Only Global + Times Needed Only Local | Average Number of Changes per Branch | Standard Deviation of Number of Changes per Branch |
| --- | --- | --- | --- |
| Go | 184,481 | 60,260.68 | 154,495 |
| M88ksim | 91,055 | 11,861.93 | 44,703 |
| Gcc | 45,978 | 11,142.8 | 13,753 |
| Compress | 1,334,968 | 556,943.35 | 1,122,678 |
| Xlisp | 217,313 | 26,129.72 | 73,481 |
| Ijpeg | 124,572 | 53,763.29 | 91,734 |
| perl | 126,049 | 17,898.75 | 54,495 |

Taking the standard deviation into account shows that although the average frequency of change may be high, the standard deviation is always higher.  This implies that there are a small number of branches who change history types very frequently while the majority of the branches do not change very often.  Compress shows the extreme case in this regard, with six branches which change history types more than 2 million times each while most of the rest only change once or twice.

# Conclusions

## Summary

Current research focuses primarily on mispredictions that arise due to resource conflicts in the branch predictor hardware.  Yet this research suggests that wrong-history mispredictions may be just as important, if not more so, than conflicts in the predictor hardware.

### Static vs. Dynamic Summary

Based on an estimation of relative performance between a static hybrid branch predictor and a dynamic hybrid branch predictor, static hybrid predictors are shown to have a significantly lower prediction accuracy.  Percentages ranged from 6 percent to a surprising 20 percent performance difference.

### Wrong-History Summary

The data show that many individual branches do change their associated history types. Branches that execute over 100,000 times were shown in most cases to be representative of well over 85 percent of the branches encountered in the SPECint95 benchmark programs.  Gcc was the only outlier with only 41 percent of the total branches being represented.  On average, of those static sites whose branches exceeded this 100,000 threshold, 74 percent changed needed-history types over the course of program execution.  Since the recorded 74 percent value does not weight the static branch sites by the number of times executed, dynamic references were also measured.  These measurements also showed that, on average, 75 percent of the total dynamic references were to static locations which changed needed history types.

### BHT Conflicts Summary

Figure 6. Percentage of time Global (GAs), Ideal Local (PAp), and Realistic Local (PAs) predicted correctly. [Drawn by Author]

Figure 20 shows that for any given benchmark program, global branch predictors are more accurate.  Also, the ideal configuration for the local branch predictor was shown to be more accurate overall than the realistic configuration, showing that aliasing in the BHT does occur.  Aliasing occurs when two branches index to the same entry slot in a branch predictor’s history tables.  Differences in prediction accuracy were more pronounced for four of the benchmarks: go, m88ksim, gcc, and perl.  Three of the benchmarks, however showed virtually no difference in the prediction accuracy: compress, xlisp, and ijpeg.  Comparing these results to those obtained by Skadron et al. [12] shows that Pattern History Table (PHT) conflicts (aliasing) are much more detrimental than conflicts in the BHT.

## Interpretation

Hybrid predictors can use either static or dynamic selection to choose which predictor component to use for each branch.  However, the changing of history types by individual branches implies that wrong-history mispredictions do cause a significant portion of the mispredictions in modern two-level branch predictors.  This project has shown that the majority of branches in the SPECint95 benchmarks do change needed history types; a catalyst for wrong-history mispredictions.

With the penalties that static hybrid predictors must pay when dealing with branches which individually change needed history types, it seems logical that static hybrid predictors should be used sparingly and in special circumstances.

This research also shows that conflicts in the BHT of local history predictors are only moderately significant for the table sizes used.  Of more significance are the conflicts in the PHT as shown in [12].

## Recommendations for Future Work

Future work in this field should focus on further characterizing wrong-history mispredictions.  This may be done by analyzing the needed history types of more than just the global and local branch predictors.  For example, adding a bimodal predictor may give some added insight as to the exact performance of the individual branches.

Another way to further characterize wrong-history mispredictions would be to look at the average run length of needed-history types as well as the standard deviation.  Comparing the average run length to the standard deviation would show whether branches are switching very rapidly between the needed-history types or whether one type gets a long run before switching.

A third suggestion for future work would be to characterize aliasing in the selector hardware of the dynamic hybrid predictor.  This could be accomplished by using varying sizes of selectors when running the simulations and comparing the performance data.  If performance varied, this would show that aliasing in the selector is a concern in modern hybrid predictor design.

## Final Word

This project has resulted in a better understanding of individual branch behaviors.  This knowledge will aid researchers when deciding the cost-benefit relationship of correcting wrong-history misprediction and to allow more accurate configurations of existing hardware, thereby increasing overall processor performance.  For example, programs with branches that require both local history and global history trackers for accurate prediction would perform faster with the hybrid type of branch predictor.

In the case of a microarchitecture with hybrid predictor capabilities, the data gathered will aid in the configuration of the selection hardware for a wide range of programs.  This project may aid the configuration of dynamic predictor hardware by providing a more in-depth analysis of each branch’s behavior, thereby allowing a better tuning of the selector for maximum performance.

Another benefit of characterizing wrong-history mispredictions is that this data can be used directly to configure newer designs such as that of the alloy predictor which uses both global and local history together at the same time to make a prediction [12].  In the case of the alloyed predictor, the data gathered will allow researchers to determine the best-performing configuration of the alloyed bits based on actual branch behavior in certain SPECint95 programs.  For example, if most of the programs had branches that needed global 60 percent of the time while needing local only 25 percent of the time, this would imply that twice as many global bits as local bits should be alloyed together when making the prediction.  (These example percentages need not add up to 100 percent since some branches are mispredicted by both global and local predictors.)

One last benefit of this project is that static hybrid predictors are better characterized.  Static hybrid predictors are shown to not be feasible for the general application.  Should designers be willing to trade predictor accuracy for size, however, this project will provide designers of such devices with high-quality training data.  The data generated could be used to configure the static predictor so that it will give the best prediction results over the widest range of programs.

Works Cited

[1]	D. Burger, T. M. Austin, and S. Bennett. Evaluating future microprocessors: the SimpleScalar tool set. Tech. Report TR-1308, Univ. of Wisconsin-Madison Computer Sciences Dept., July 1996.

[2]	P.-Y. Chang, E. Hao, and Y. N. Patt. Alternative implementations of hybrid 	branch 	predictors. Proceedings of the 28th International Symposium on 	Microarchitecture, pages 252-57, Dec. 1995.

[3]	A. N. Eden and T. Mudge. The YAGS branch prediction scheme. Proceedings of 	the 31st International Symposium on Microarchitecture, pages 69-77, Dec. 1998.

[4]	D. Grunwald, D. Lindsay, and B. Zorn. Static methods in hybrid branch 	prediction. Proceedings of the 1998 International Conference on Parallel 	Architectures and Compilation Techniques, pages 222-29, Oct. 1998.

[5]	V. P. Heuring and H. F. Jordan.  Computer Systems Design and Architecture. Addison Wesley Longman, Inc. Pages 195-227, 1997.

[6]	N. P. Jouppi and P. Ranganathan.  The relative importance of memory latency, 	bandwidth, and branch limits to performance.  In The Workshop on Mixing Logic 	and DRAM: Chips that Computer and Remember, June 1997.  	http://ayer.cs.berkeley.edu/isca97-workshop.

[7]	C.-C. Lee, I.-C.K. Chen, and T.N. Mudge. The bi-mode branch predictor. In Proceedings of the 30th International Symposium on Microarchitecture, pages 4-13, Dec. 1997.

[8]	S. McFarling. Combining branch predictors.  Tech. Note TN-36, Compaq Western Research Laboratory, June 1993.

[9]	P. Michaud, A. Seznec, and R. Uhlig. Trading conflict and capacity aliasing in 	conditional branch predictors. In Proceedings of the 24th International 	Symposium on Computer Architecture, pages 292-303, June 1997.

[10]	S. Sechrest, C.-C. Lee, and T. Mudge. Correlation and aliasing in dynamic branch 	predictors. In Proceedings of the 23th International 	Symposium on Computer 	Architecture, pages 22-32, May 1995.

[11]	K. Skadron.  CS551/851: "Advanced Topics in Computer Architecture: A Microprocessor Survey." Dec. 1999. http://www.cs.virginia.edu/~skadron/cs851.

[12]	K. Skadron, M. Martonosi, and D.W. Clark.  "Alloying Global and Local Branch History: A Robust Solution to Wrong-History Mispredictions."  Tech Report TR-606-99, Princeton Dept. of Computer Science, Oct. 1999.  Submitted for publication.

[13]	E. Sprangle, R. S. Chappell, M. Alsup, and Y.N. Patt. The agree predictor: A 	mechanism for reducing negative branch history interference. In Proceedings of 	the 24th International Symposium on Computer Architecture, pages 284-91, June 	1997.

[14]	The Standard Performance Evaluation Corporation.  WWW Site. 	http://www.specbench.org, Dec. 1996.

[15]	T.-Y. Yeh and Y. N. Patt. A comparison of dynamic branch predictors that use 	two levels of branch history. In Proceedings of the 20th International Symposium 	on Computer Architecture, pages 257-66, May 1993.

Appendix A.  Per-Branch Data for Realistic and Ideal branch predictor configurations.

Benchmark: Go

Figure 7. Go: Per-Branch Data for Ideal Configuration 
				[Drawn by author].

Figure 8. Go: Per-Branch Data for Realistic Configuration 
				[Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 102781.17 | 287542.63 |
| Local | 77856.93 | 149511.89 |
| Changes | 56113.52 | 154273.69 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 108065.55 | 287468.63 |
| Local | 76416.25 | 149314.94 |
| Changes | 60260.68 | 154495.57 |

Benchmark: M88ksim

Figure 9. M88ksim: 	Per-Branch Data for Ideal Configuration 							[Drawn by author].

Figure 10. M88ksim:  Per-Branch Data for Realistic 								  Configuration [Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 44168.11 | 268053.89 |
| Local | 25208.62 | 72630.17 |
| Changes | 8316.50 | 20815.43 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 67044.02 | 278098.58 |
| Local | 24011.35 | 71511.09 |
| Changes | 11861.93 | 44703.03 |

Benchmark: Gcc

Figure 11. Gcc: Per-Branch Data for Ideal Configuration 						   [Drawn by author].

Figure 12. Gcc: Per-Branch Data for Realistic Configuration 						   [Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 29188.99 | 48414.02 |
| Local | 14126.34 | 12992.53 |
| Changes | 9793.22 | 13951.57 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 32143.83 | 48134.45 |
| Local | 13835.04 | 12871.71 |
| Changes | 11142.80 | 13753.82 |

Benchmark: Compress

Figure 13. Compress: Per-Branch Data for Ideal Configuration 						  [Drawn by author].

Figure 14. Compress: Per-Branch Data for Realistic Configuration 						  [Drawn by author].

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 776536.88 | 1596518.01 |
| Local | 558432.88 | 943822.93 |
| Changes | 556943.35 | 1122678.01 |

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 776536.82 | 1596518.09 |
| Local | 558432.79 | 943822.92 |
| Changes | 556943.29 | 1122678.04 |

Benchmark: Xlisp

Figure 15. Xlisp: Per-Branch Data for Ideal Configuration 						     [Drawn by author].

Figure 16. Xlisp: Per-Branch Data for Realistic Configuration 						     [Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 50235.04 | 159163.75 |
| Local | 165566.76 | 737328.10 |
| Changes | 25577.00 | 73442.79 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 166806.82 | 736054.28 |
| Local | 50507.17 | 159107.13 |
| Changes | 26129.72 | 73481.99 |

Benchmark: Ijpeg

Figure 17. Ijpeg: Per-Branch Data for Ideal Configuration 						     [Drawn by author].

Figure 18. Ijpeg: Per-Branch Data for Realistic Configuration 						     [Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 64456.05 | 115972.18 |
| Local | 60121.06 | 88365.03 |
| Changes | 53764.08 | 91737.16 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 60121.52 | 88365.25 |
| Local | 64451.46 | 115970.46 |
| Changes | 53763.29 | 91734.76 |

Benchmark: Perl

Figure 19. Perl: Per-Branch Data for Ideal Configuration 						    [Drawn by author].

Figure 20. Perl: Per-Branch Data for Realistic Configuration 						    [Drawn by author].

| IDEAL | Average | Std Dev |
| --- | --- | --- |
| Global | 79544.99 | 388810.22 |
| Local | 35070.62 | 65824.00 |
| Changes | 15931.62 | 52172.08 |

| REAL | Average | Std Dev |
| --- | --- | --- |
| Global | 90999.55 | 390475.76 |
| Local | 35050.72 | 65444.61 |
| Changes | 17898.75 | 54495.22 |

Italicized words are defined in the Glossary of Terms on page V .

The program counter is a special register where the processor keeps the memory address of the current instruction [5].

A brief note on nomenclature: needed-history type will be used in the remainder of this report to refer to the type of branch predictor that performs best for a given branch.  For example, if a local predictor out-performs a global predictor for branch A, then branch A is said to need a local history type.

All tables in report are drawn by author.